Open Edge

Intelligent Desktop Virtualization (IDV)

Revision 0.4.0

Date: 8/12, 2020

RBHE Transactional

Philip Park

Joko Sastriawan

Contents

1	Intelligent Desktop Virtualization (IDV)	5

1.1	History	5

1.2	Architecture overview	5

1.3	Pre-requisite	6

1.3.1	Ubuntu 18.04	6

1.3.2	Centos 7	7

2	Build Base OS	8

2.1	Build kernel with IDV patch	8

2.1.1	Build the Initrd (Initial Ramdisk)	8

2.1.1.1	Ubuntu OS	8

2.1.1.2	Centos OS	9

2.1.2	Build kernel source	9

2.1.2.1	With idv3.0 patches	9

2.1.2.2	With idv-2.1 patches	12

2.2	Install and configure kernel	14

2.2.1	Installation of the kernel	14

2.2.2	Configure grub to boot newly installed kernel	14

2.2.3	GVTg Verification	15

2.2.3.1	Check for “mdev_supported_types “	16

2.2.3.1.1	i915_GVTg_V5_1	16

2.2.3.1.2	i915_GVTg_V5_2	17

2.2.3.1.3	i915_GVTg_V5_4	17

2.2.3.1.4	i915_GVTg_V5_8	17

3	QEMU build	17

3.1	Install qemu on LTS 20.04	17

3.2	Install QEMU on LTS 18.04	18

3.2.1	Install Package to build QEMU	18

3.2.2	Download, configure QEMU, compile to install	18

4	Putting it together	20

4.1	Recommended directory structure	20

4.1.1	“disk” directory	20

4.1.2	“fw” directory	21

4.1.3	“iso” directory	22

4.2	Available GVTg control node	22

4.2.1	Explanation of gvt_disp_ports_status	22

4.2.2	Explanation of gvt_disp_ports_mask	23

4.2.3	Explanation of gvt_disp_ports_owner	24

4.3	Network Preparation	24

4.4	Graphics pass through	25

4.4.1	Create UUID (GUID)	25

4.4.2	Masking the ports for VGPU	25

4.4.3	Assigning the port/s to vGPU using UUID	25

4.4.4	De-coupling the ports	26

5	Installing guest OS	26

5.1	Start VM with USB device	26

5.2	Advanced topic	27

5.2.1	Network in NAT mode	27

5.2.2	Utilization of system	27

5.2.3	Display Switch between Host and VM	28

5.2.4	Device pass-through/assignment to guest OS	29

5.2.4.1	Passing USB by Bus and Device	29

5.2.4.2	Passing USB by vendor and product ID	29

5.2.4.3	Passing by USB port	30

5.2.4.4	Passing by PCI device using vfio-pci	31

5.2.4.5	Passthrough MISC devices	32

5.2.5	Headless VM setup for Spice	32

5.2.6	Automating VM deployment using Portainer	33

5.2.7	Automating VM deployment using LibVirt	33

6	Remote management using MeshCentral	34

6.1	Basic usage of MeshCentral	34

6.2	Advance features with MeshCentral	36

Addendum	38

Scripts to pull and build kernels	38

Script to pull and build qemu	39

VFIO PCI unbind and bind script	40

Gvt.sh (create-vgpu, setup directory)	41

Use case	43

QEMU parameters	43

# Intelligent Desktop Virtualization (IDV)

TBD Introduction: IDV, VTx, VTd,

## History

| Revision | Date | Note |
| --- | --- | --- |
| 0.2 | 2/27, 2020 | Draft for Ubuntu 18.04 |
| 0.2.1 | 2/28, 2020 | Update the document per Jimmy’s input (70% done) |
| 0.2.2 | 3/2, 2020 | Update the document per Jimmy’s input-done. |
| 0.2.3 | 3/3, 2020 | Update USB assignment method in Advance section Add MeshCentral with advance features |
| 0.3.0 | 3/10, 2020 | Add gvt.sh setup file to automate the create-vgpu and vm directory structure |
| 0.4.0 | 8/12, 2020 | Add section 3.1 to support LTS 20.04 Add IDV 3.0 patch support (section 2, Build Base OS) Add MISC device passthrough bkm |

## Architecture overview

IDV solution can co-exist AIC-CIC and AIC-CIV.

## Pre-requisite

IDV has been tested with Ubuntu 18.04 and Centos7

Bootable USB

Make bootable USB thumb drive with ISO. Rufus can be used to make bootable drive. Recommended size for USB thumb drive is greater than 8G.

Download Rufus from https://rufus.ie/.

### Ubuntu 18.04

Ubuntu 18.04 text mode is recommended. However, Ubuntu 18.04 minimal GUI is alternative.

Mini.iso

User can download the Ubuntu 18.04 text mode “mini.iso” from mini.iso

Minimal GUI installation option with normal Ubuntu 18.04 distro

User can download the Ubuntu 18.04 desktop ISO file from the following link.

https://ubuntu.com/download/desktop

Please select “Minimal Installation” and “development package” option while installing Ubuntu.

### Centos 7

Download the ISO image from http://mirror.centos.org/centos/7/os/x86_64/.

Tested with minimal installation with development package.

TBD

Hardware requirement

The IDV package currently supports KBL/SKL/CFL. Number of VM is depends on the memory size, number of threads in CPU, and application you want to run on each VM.

Supported graphics port is : HDMI/DP/DVI

VGA port is not supported.

General rule of thumb is as below. Please note that implementer can experiment the allocation as 1 thread/2G memory also works depends on your application.

| OS | Thread | Memory |
| --- | --- | --- |
| Windows 10 | 2 | > 4G |
| Ubuntu | 2 | > 2G |
| Android | 2 | > 3G |

Aperture size in BIOS setup

Aperture size in BIOS setup needs to be at least 512. The recommend setting will be maximum size available above 512.

# Quick start guide

The semi-automated script will be used in this section for quick start. Please contact your local field intel engineers to get reference scripts.

| Build-kernel.sh | Script to build the kernel and install. Modifies kernel environment to accommodate gvt-g environment with newly build kernel |
| --- | --- |
| Config.sh | Configure guest OS in /var/vm directory |
| Install-guest-os.sh | Help to install the guest OS |
| Start-geust-os.sh | Help to validate installed OS |

## Build kernel using script

When this script ran first time, it will go through the menu to establish the environment and source of the kernel.

The “.idv_config” file will be created in “idv” directory. You can manually delete the “.idv_config” file to start over.

The following screen will select the appropriate kernel repo. Recommended to use the IOTG-repo which available as public access.

The script will automatically pull the kernel source and start building in docker image.

## Install kernel

Please install kernel using the option provide once kernel build successfully. It will configure the grub as part of the installation.

## Configuration using config.sh file

The following screen shows up when you run the “config.sh” file.

### mdev option

The options showing in this step is based on the memory allocated for GFX aperture. If enough memory is allocated, higher resolution option should be available. The default selection is for “1920x1080” resolution (I915-GVTg_V5_4).

If available aperture size is set to small then this option will not show.

### Assigning each GFX port to guest OS

The port options will show up depends on how monitors are connected to the system. The following “PORT_B”, “PORT_C”, and “PORT_D” for a system with 3 monitors are connected. If no monitor is connected, the “config.sh” script will abort with message.

- Select desired port (e.g. PORT_B, PORT_C, or PORT_D)

- Select guest OS ISO file. In this example, Windows are selected.

- Select the firmware option

  - For Windows, please select bios.bin file for stability

- J

- K

- L

# Build GVTg enable OS and Configure for VM

Automated scripts are provided to aid

Please install Ubuntu LTS 18.04/20.04 in either text/minimal GUI mode on bare metal.

## Build kernel with IDV patch

Provided “build-kernel.sh” supports two kernel repositories. For now, it is recommended to use the Vanilla repository. When executed first time, the following dialog open for user to select source of kernel repository.

What it does:

Install docker

Create docker image

Install packages needed to compile the kernel in docker image

Modify the /etc/initramfs-/modules

Modify the grub file when installing newly built kernel

- Vanilla

Vanilla is public kernel source repo. You can access this repo from any IP address.

- IOTG-repo

IOTG-repo is internal to intel. Can’t access from public IP address. No patches is supplied if this option is selected.

Select patches

Click <OK> to start build the kernel

## Configure the VM

Provided “config.sh” will configure your system to be able to run VM.

The config.sh file expects all guest OS ISO file located in “iso” directory. Download the guest OS ISO files before running “config.sh”.

/var/vm/ directory gets populated with scripts and files.

At this point, the guest OS disk is not created.

Need to run the install-guest-os.sh <option>

Need to reboot

Cd /var/vm/scripts/create-vgpu.sh

Sudo ./create-vgpu.sh

Install-guest.sh vgpuX

Start the VNC viewer to anticipate the user options during guest OS installation

Please note that display driver is not loaded. It only shows MS basic display adapter driver.

This is because the installation script is not connected to a GFX port via uuid. It only shows up on vncviewer.

Now you can shutdown the guest OS.

Note: you might want to use keyboard through-out the windows installation if mouse is not working

# Manual Build

Please install Ubuntu LTS 18.04/20.04 in either text/minimal GUI mode on bare metal.

## Build Kernel

### Build the Initrd (Initial Ramdisk)

The following is the list of modules needs to be added to kernel

- kvmgt

- vfio-iommu-type1

- vfio-mdev

- vfio-pci (optional. It is for PCI->USB cards pass through)

#### Ubuntu OS

Modify the /etc/initramfs-tools/modules as root to add the following modules.

Please refer to the following example.

Example of modules file 1 # List of modules that you want to include in your initramfs. # They will be loaded at boot time in the order below. # # Syntax: module_name [args ...] # # You must run update-initramfs(8) to effect this change. # # Examples: # # raid1 # sd_mod kvmgt vfio-iommu-type1 vfio-mdev vfio_pci

After saving /etc/initramfs-tools/modules, please execute the following command.

# execute following command to update $ sudo update-initramfs -u

#### Centos OS

Modify the /etc/dracut.conf file to add the modules before building the kernel

# additional kernel modules to the default Add_drivers+="kvmgt vfio-iommu-type1 vfio-mdev"

TBD

### Build kernel source

The following will pull the appropriate kernel branch and start kernel deb package.

For easy of documenting purpose, the following alias is used.

#### With idv3.0 patches

| Alias | Note |
| --- | --- |
| CDIR | current working root directory (recommend ~/build as your working directory) CDIR=$(pwd) |
| URL | URL where kernel source can be download URL="https://git.kernel.org/pub/scm/linux/kernel/git/stable/linux.git" |
| BRANCH | tag or branch for kernel tree in git BRANCH="v5.4.54" |
| WDIR | working directory where kernel source will be downloaded to WDIR="idv-3" |
| PATCHES | name of the patches you download from here. https://platformsw.intel.com/ddrivers.aspx?kitnumber=135055 PATCHES="idv3_er3_patchset" |

Note: subject to change as URL/branch/patch files are changing

Building kernel involves following steps.

- Install the packages

The following package is needed to build kernel. It is possible that additional package is needed depends on the installed OS. The following is tested with Ubnuntu 18.04 installation with “minimal GUI” option.

Example of modules file 2 Packages needed # Install packages needed $ sudo apt-get install -y liblz4-tool kernel-package libelf-dev build-essential git libfdt-dev libpixman-1-dev libssl-dev vim bc socat libsdl1.2-dev libspice-server-dev autoconf libtool xtightvncviewer tightvncserver x11vnc uuid-runtime uuid uml-utilities bridge-utils python-dev liblzma-dev libc6-dev libegl1-mesa-dev libepoxy-dev libdrm-dev libgbm-dev spice-client-gtk libgtk2.0-dev libusb-1.0-0-dev bison flex openssh-server net-tools

- Update the package and upgrade

It is recommended to update and upgrade the packages with latest.

# Update the packages and upgrade $ sudo apt-get update && sudo apt-get upgrade

- Pull the kernel tree with correct tag or branch

The following command will connect to $URL git kernel repository to pull the specific tag or branch indicated in $BRANCH into $WDIR. The following command will create $WDIR that user doesn’t have to create.

### IDV 2.1 section 3.2.2 Build Kernel Source git clone --depth 1 $URL --branch $BRANCH --single-branch $WDIR

- Apply the patches

# unzip download patched # Unzip downloaded patches tar xzvf $CDIR/$PATCHES.tar.gz cd $WDIR git apply $CDIR/$PATCHES/* &>/dev/null

- Configure the kernel

# Create configuration file echo ""¦make oldconfig # Verify the kernel configuration is done correct or check the # contents by opening the .config file using text editor egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config # Make sure you see the following output Table 1 Output of "egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config dev@build:~/work/idv21/idv-2.1$ egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config CONFIG_DRM_I915_GVT=y CONFIG_DRM_I915_GVT_KVMGT=m CONFIG_VFIO_MDEV=m CONFIG_VFIO_MDEV_DEVICE=m dev@build:~/work/idv21/idv-2.1$.

- Compile the kernel source with overlay method

The following command will generate deb package with version and revision code.

# The following will compile the code can create deb package $ CONCURRENCY_LEVEL=`nproc` fakeroot make-kpkg --initrd --append-to-version=-intelgvt --revision 2.1.0 --overlay-dir=$cdir/ubuntu-package kernel_image kernel_headers

linux-image-4.19.65-intelgvt+_2.1.0_amd64.deb

linux-headers-4.19.65-intelgvt+_2.1.0_amd64.deb

  - –append-to-version=-intelgvt

  - –revisioin 2.1.0

Where 2.1 is IDV package version and 0 is engineering release version

#### With idv-2.1 patches

| Alias | Note |
| --- | --- |
| CDIR | current working root directory (recommend ~/build as your working directory) CDIR=$(pwd) |
| URL | URL where kernel source can be download URL="https://git.kernel.org/pub/scm/linux/kernel/git/stable/linux.git" |
| BRANCH | tag or branch for kernel tree in git BRANCH="v4.19.65" |
| WDIR | working directory where kernel source will be downloaded to WDIR="idv-2.1" |
| PATCHES | name of the patches you download from here. https://platformsw.intel.com/ddrivers.aspx?kitnumber=135055 PATCHES="idv2.1_er5_patchset" |

Note: subject to change as URL/branch/patch files are changing

Building kernel involves following steps.

- Install the packages

The following package is needed to build kernel. It is possible that additional package is needed depends on the installed OS. The following is tested with Ubnuntu 18.04 installation with “minimal GUI” option.

Example of modules file 3 Packages needed # Install packages needed $ sudo apt-get install -y liblz4-tool kernel-package libelf-dev build-essential git libfdt-dev libpixman-1-dev libssl-dev vim bc socat libsdl1.2-dev libspice-server-dev autoconf libtool xtightvncviewer tightvncserver x11vnc uuid-runtime uuid uml-utilities bridge-utils python-dev liblzma-dev libc6-dev libegl1-mesa-dev libepoxy-dev libdrm-dev libgbm-dev spice-client-gtk libgtk2.0-dev libusb-1.0.0-dev bison flex openssh-server net-tools

- Update the package and upgrade

It is recommended to update and upgrade the packages with latest.

# Update the packages and upgrade $ sudo apt-get update && sudo apt-get upgrade

- Pull the kernel tree with correct tag or branch

The following command will connect to $URL git kernel repository to pull the specific tag or branch indicated in $BRANCH into $WDIR. The following command will create $WDIR that user doesn’t have to create.

### IDV 2.1 section 3.2.2 Build Kernel Source git clone --depth 1 $URL --branch $BRANCH --single-branch $WDIR

- Apply the patches

# unzip download patched # Unzip downloaded patches tar xzvf $CDIR/$PATCHES.tar.gz cd $WDIR git apply $CDIR/$PATCHES/* &>/dev/null

- Configure the kernel

# Create configuration file echo ""¦make oldconfig # Verify the kernel configuration is done correct or check the # contents by opening the .config file using text editor egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config # Make sure you see the following output Table 2 Output of "egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config dev@build:~/work/idv21/idv-2.1$ egrep 'CONFIG_DRM_I915_GVT¦CONFIG_VFIO_MDEV' .config CONFIG_DRM_I915_GVT=y CONFIG_DRM_I915_GVT_KVMGT=m CONFIG_VFIO_MDEV=m CONFIG_VFIO_MDEV_DEVICE=m dev@build:~/work/idv21/idv-2.1$.

- Compile the kernel source with overlay method

The following command will generate deb package with version and revision code.

# The following will compile the code can create deb package $ CONCURRENCY_LEVEL=`nproc` fakeroot make-kpkg --initrd --append-to-version=-intelgvt --revision 2.1.0 --overlay-dir=$cdir/ubuntu-package kernel_image kernel_headers

linux-image-4.19.65-intelgvt+_2.1.0_amd64.deb

linux-headers-4.19.65-intelgvt+_2.1.0_amd64.deb

  - –append-to-version=-intelgvt

  - –revisioin 2.1.0

Where 2.1 is IDV package version and 0 is engineering release version

## Install and configure kernel

### Installation of the kernel

The following commands will install “linux-image-4.19.65-intelgvt+_2.1.0_amd64.deb” and “

linux-headers-4.19.65-intelgvt+_2.1.0_amd64.deb”.

# the following command will install the kernel cd $CDIR sudo dpkg -i *.deb

### Configure grub to boot newly installed kernel

In Linux system, you can have multiple revisions of OS that you can selectively boot from. This example shows how GRUB can be used to boot. The default GRUB file is located in “/etc/default/grub”. We are interested in two options.

| GRUB_DEFAULT | Default OS version to be boot from |
| --- | --- |
| GRUB_CMDLINE_LINUX_DEFAULT | Options to pass to OS |

- Edit the “/etc/default/grub” as root and make modification to the file as below.

# to modify the /etc/default/grub as root $ sudo nano /etc/default/grub

Modify the “GRUB_DEFAULT” and “GRUB_CMDLINE_LINUX_DEFAULT”

GRUB_DEFAULT="Advanced options for Ubuntu>Ubuntu, with Linux 4.19.65-intelgvt+" GRUB_CMDLINE_LINUX_DEFAULT="splash quiet i915.enable_gvt=1 i915.enable_fbc=0 kvm.ignore_msrs=1 intel_iommu=on,igfx_off drm.debug=0"

Note: 4.19.65-intelgvt is part of the kernel deb file name. Upon installation of the kernel, this string should be in /boot/grub/grub.cfg file.

- Apply the default grub setting

Must run the following command to have it take new rules. It will take effect on next reboot

# to modify the /etc/default/grub as root $ sudo update-grub2

- Verification of the new kernel after reboot

Upon reboot, all GRUB setting should take effect and boot to new kernel

# reboot system $ sudo reboot

# Check the kernel and options $ cat /proc/cmdline # Please note that “BOOT_IMAGE” reflects the compiled and installed image and options $ cat /proc/cmdline BOOT_IMAGE=/boot/vmlinuz-4.19.65-intelgvt+ root=UUID=baea869f-283c-483e-b29e-8ea87927b48c ro splash quiet i915.enable_gvt=1 i915.enable_fbc=0 kvm.ignore_msrs=1 intel_iommu=on,igfx_off drm.debug=0 vt.handoff=1

### GVTg Verification

#### Check for “mdev_supported_types “

Once reboot with correct build, grub configuration, and reboot, “medv_supported_types” directory should exist. Depends on your aperture size, you will see one or more of the following directories under “mdev_supported_types”.

- i915_GVTg_V5_1

- i915_GVTg_V5_2

- i915_GVTg_V5_4

- i915_GVTg_V5_8

# Check for mdev_supported_types directory $ ll /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/ # The output of “ll /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/” Table 3 Output of “ll” command for mdev_supported_type directory dev@gvt:~$ ll /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/ total 0 drwxr-xr-x 6 root root 0 Feb 28 13:09 ./ drwxr-xr-x 14 root root 0 Feb 24 14:55 ../ drwxr-xr-x 3 root root 0 Feb 24 14:55 i915-GVTg_V5_1/ drwxr-xr-x 3 root root 0 Feb 24 14:55 i915-GVTg_V5_2/ drwxr-xr-x 3 root root 0 Feb 24 14:55 i915-GVTg_V5_4/ drwxr-xr-x 3 root root 0 Feb 24 14:55 i915-GVTg_V5_8/ dev@gvt:~$.

##### i915_GVTg_V5_1

# Check for mdev_supported_types directory $ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_1 # Output of above command Table 4 Output of Description of node i915_GVTg_V5_1 giga@giga-0t:~$ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_1/description low_gm_size: 512MB high_gm_size: 2048MB fence: 4 resolution: 1920x1200 weight: 16.

##### i915_GVTg_V5_2

# Check for mdev_supported_types directory $ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_2 # Output of above command Table 5 Output of Description of node i915_GVTg_V5_2 giga@giga-0t:~$ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_2/description low_gm_size: 512MB high_gm_size: 2048MB fence: 4 resolution: 1920x1200 weight: 16.

##### i915_GVTg_V5_4

# Check for mdev_supported_types directory $ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_4 # Output of above command Table 6 Output of Description of node i915_GVTg_V5_4 giga@giga-0t:~$ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_4/description low_gm_size: 512MB high_gm_size: 2048MB fence: 4 resolution: 1920x1200 weight: 16.

##### i915_GVTg_V5_8

# Check for mdev_supported_types directory $ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_8 # Output of above command Table 7 Output of Description of node i915_GVTg_V5_8 giga@giga-0t:~$ cat /sys/bus/pci/devices/0000:00:02.0/mdev_supported_types/i915_GVTg_V5_8/description low_gm_size: 512MB high_gm_size: 2048MB fence: 4 resolution: 1920x1200 weight: 16.

# QEMU build

## Install qemu on LTS 20.04

If you have LTS 20.04 as base OS then use method in this section.

- Install qemu package for LTS 20.04. It will install qemu and BIOS.bin file.

- Copy the bios.bin and OVMF.fd file to “vm” work direcotry

cp /usr/share/qemu/bios.bin ~/vm/fw cp /usr/share/qemu/OVMF.fd ~/vm/fw

## Install QEMU on LTS 18.04

If you are using 18.04 and follow the method in this section.

### Install Package to build QEMU

# Install packages to build QEMU $ sudo apt install -y git libfdt-dev libpixman-1-dev libssl-dev vim socat libsdl2-dev libspice-server-dev autoconf libtool xtightvncviewer tightvncserver x11vnc uuid-runtime uuid uml-utilities bridge-utils python-dev liblzma-dev libc6-dev libegl1-mesa-dev libepoxy-dev libdrm-dev libgbm-dev libaio-dev libusb-1.0.0-dev libgtk-3-dev bison

### Download, configure QEMU, compile to install

# Pull the tree $ wget https://download.qemu.org/qemu-4.2.0.tar.xz # Un-compress the tar ball file $ tar xzvf qemu-4.2.2.tar.gz # change directory to qemu source tree $ cd qemu-4.2.2 # pull submodules <optional> $ git submodule update --init roms/seabios # configure the QEMU $ ./configure --prefix=/usr \ --enable-kvm \ --disable-xen \ --enable-libusb \ --enable-debug-info \ --enable-debug \ --enable-sdl \ --enable-vhost-net \ --enable-spice \ --disable-debug-tcg \ --enable-opengl \ --enable-gtk \ --target-list=x86_64-softmmu \ --audio-drv-list=pa # make source tree $ make -j `nproc` # build bios.bin (optional) $ cd roms/seabios $ make -j`nproc` $ cd - # Install the QEMU sudo make install # copy the bios.bin to vm/fw directory (optional) $ cp roms/seabios/out/bios.bin ~/vm/fw

-------------------------------------------------------------------------------

The kernel was built with IDV patches. User can verify the GVTg.

Once should be able to see “gvt_disp_auto_switch”, “gvt_disp_edid_filter”, “gvt_disp_ports_mask”, “gvt_disp_ports_owner”, and “gvt_disp_ports_status”.

# Putting it together

## Recommended directory structure

Recommend the following directory setup as below for easy maintenance. This document and scripts as example assume the following directory is exist and refers to them. However, one can change.

| Directory | Note |
| --- | --- |
| vm | parent directory consisting fw, disk, iso, and script |
| fw | directory where firmware is located |
| disk | directory where collection of qcow2 files |
| iso | directory where IOS image |
| script | directory where all scripts are located |

The recommended directory structures. Please see rest of document for explanation.

### “disk” directory

The “disk” directory is where guest OS is located in “qcow2” file format.

# create qcow2 file where guest OS to be installed $ qemu-img create -f qcow2 ubuntu18.qcow2 60G $ qemu-img create -f qcow2 win10.qcow2 60G

### “fw” directory

The “fw” directory is where BIOS and UEFI firmware is located.

The bios.bin is results of part of QEMU build. Please see 3.2.2.

The requirements of bios.bin is depends on the guest OS. If the guest OS is needs UEFI, then the bios.bin is not needed.

# 1) The following link has the pre-build OVMF files where you can download # https://www.kraxel.org/repos/jenkins/edk2/ #----------------------------------------------------- # 2) Download the pre-build OVMF rpm package. Please note that the file name # changes as it gets updated. If it fails, you can select the correct file name # by viewing following link on web # “https://www.kraxel.org/repos/jenkins/edk2/” #----------------------------------------------------- $ wget https://www.kraxel.org/repos/jenkins/edk2/edk2.git-ovmf-x64-0-20191118.1337.g2be4828af1.noarch.rpm #----------------------------------------------------- # 3) Unpack the rpm package to current direction without installing them #----------------------------------------------------- $ rpm2cpio edk2.git-ovmf-x64-0-20191118.1337.g2be4828af1.noarch.rpm ¦ cpio -idmv $ rpm2cpio edk2.git-ovmf-x64-0-20191118.1337.g2be4828af1.noarch.rpm ¦ cpio -idmv ./usr/share/doc/edk2.git-ovmf-x64 ./usr/share/doc/edk2.git-ovmf-x64/README ./usr/share/edk2.git ./usr/share/edk2.git/ovmf-x64 ./usr/share/edk2.git/ovmf-x64/OVMF-need-smm.fd ./usr/share/edk2.git/ovmf-x64/OVMF-pure-efi.fd Tried and worked ./usr/share/edk2.git/ovmf-x64/OVMF-with-csm.fd Tried and worked ./usr/share/edk2.git/ovmf-x64/OVMF_CODE-need-smm.fd ./usr/share/edk2.git/ovmf-x64/OVMF_CODE-pure-efi.fd ./usr/share/edk2.git/ovmf-x64/OVMF_CODE-with-csm.fd ./usr/share/edk2.git/ovmf-x64/OVMF_VARS-need-smm.fd ./usr/share/edk2.git/ovmf-x64/OVMF_VARS-pure-efi.fd ./usr/share/edk2.git/ovmf-x64/OVMF_VARS-with-csm.fd ./usr/share/edk2.git/ovmf-x64/UefiShell.iso ./usr/share/qemu/firmware/80-ovmf-x64-git-needs-smm.json ./usr/share/qemu/firmware/81-ovmf-x64-git-pure-efi.json ./usr/share/qemu/firmware/82-ovmf-x64-git-with-csm.json $ #----------------------------------------------------- # 4) Copy the OVM files to “fw” directory #----------------------------------------------------- $ cp ./usr/share/edk2.git/ovmf-x64/OVMF-pure-efi.fd ~/vm/fw/OVMF.fd $ cp ./usr/share/edk2.git/ovmf-x64/OVMF-with-csm.fd ~/vm/fw/OVMF-csm.fd

### “iso” directory

Please consult your local IT for a copy of Windows 10 ISO file

Please where you can download the IOS file. OS; Link. Ubuntu 18.04; https://ubuntu.com/download/desktop. Android x86; https://osdn.net/projects/android-x86/releases.

## Available GVTg control node

# Check the GVTg control node. $ cat /sys/class/drm/card0/gvt* # Output should be as below Table 8Output of "ls /sys/class/drm/card0/gvt*" $ ls /sys/class/drm/card0/gvt* /sys/class/drm/card0/gvt_disp_auto_switch /sys/class/drm/card0/gvt_disp_edid_filter /sys/class/drm/card0/gvt_disp_ports_mask /sys/class/drm/card0/gvt_disp_ports_owner /sys/class/drm/card0/gvt_disp_ports_status.

### Explanation of gvt_disp_ports_status

Shows current status of GVTg.

The following example indicates there are 3 ports available and mapped.

# Check the port status of GVTg $ cat /sys/class/drm/card0/gvt_disp_ports_status $ cat /sys/class/drm/card0/gvt_disp_ports_status Auto host/vGPU display switch: Y Available display ports: 0x00004320 Ports available ( PORT_B(2) ) ( PORT_C(3) ) ( PORT_D(4) ) Display ports assignment: 0x0000000000080402 Currently mapped Each byte represents the bit mask of assigned port(s) to vGPU 1-8 (low to high) Bit mask is decoded from LSB to MSB (PORT_A to PORT_A+7): 0: This port isn't assigned to that vGPU. 1: This port is assigned to that vGPU. vGPU-1 port mask: (0x02) ( PORT_B(2) ) vGPU-2 port mask: (0x04) ( PORT_C(3) ) vGPU-3 port mask: (0x08) ( PORT_D(4) ) Display ports ownership: 0x00000000 Ownership of the display port x on n-th hex-digit: (port_name(port_id#) <---> owner) n: port id, PORT_A(1), PORT_B(2), ... x: owner: host (0), vGPU id(x) ( PORT_B(2) <---> Host ) ( PORT_C(3) <---> Host ) ( PORT_D(4) <---> Host )

### Explanation of gvt_disp_ports_mask

“gvt_disp_ports_mask” can accommodate up to 8 physical monitor assignment each with 8 bits to indicates graphics ports. The vGPU(x) is mapped using bitmapped definition.

IDV 2.1 gvt_disp_ports_mask definition – each xx is OR-able 8bit GPU index; vGPU7; vGPU6; vGPU5; vGPU4; vGPU3; vGPU2; vGPU1; vGPU0. Pipe ownership mapping; xx; xx; xx; xx; xx; xx; xx; xx. Where xx is defined as below and can be OR-ed PORT; BITMASK; HEX. PORT A (EDP); 0000 0001; 01. PORT B; 0000 0010; 02. PORT C; 0000 0100; 04. PORT D; 0000 1000; 08. Mapping port B to vGPU0, port C to vGPU1, and port D to vGPU2 GPU index; vGPU7; vGPU6; vGPU5; vGPU4; vGPU3; vGPU2; vGPU1; vGPU0. Pipe ownership mapping; xx; xx; xx; xx; xx; 08; 04; 02. Mapping port B & C to vGPU0, and port D to vGPU2 GPU index; vGPU7; vGPU6; vGPU5; vGPU4; vGPU3; vGPU2; vGPU1; vGPU0. Pipe ownership mapping; xx; xx; xx; xx; xx; xx; 08; 06.

### Explanation of gvt_disp_ports_owner

This feature is implemented to accommodate single monitor with VM and AIC-CIC running at same time by giving framebuffer switching. The detail will be TBD.

Gvt_disp_port_ownership is total 32bit field with each nibble represents on vGPU. GPU index; vGPU7; vGPU6; vGPU5; vGPU4; vGPU3; vGPU2; vGPU1; vGPU0. gvt_disp_port_ownership; x; x; x; x; x; x; x; x. Each nibble can be either 1 or 0. The nibble will be set to “1” to guest OS while guest OS is running. It will be set to “0” when guest OS exist.

## Network Preparation

For network configuration for VM, NAT method will used in this case.

NAT mode

Create a “vm/script/qemu-ifup-nat” file and past the script indicated in the https://wiki.qemu.org/Documentation/Networking/NAT.

## Graphics pass through

### Create UUID (GUID)

Need to use unique ID to logically connect to virtual graphics pipe. “uuidgen” will be used. Generate as many as UUID needed. One UUID for one graphics port. If the system has 3 port/monitor, you will need 3 UUID. Repeat the “uuidgen” to get additional UUID (GUID).

# Generate unique id with uuidgen util $ uuidgen $ uuidgen d09054ed-30cd-474f-b662-10be479ac004

### Masking the ports for VGPU

The port needs to be logically assigned by writing “gvt_disp_ports_mask”. The 0x0000000000080402 correspondent to the UUID writes to the “create”.

# for IDV 2.1 or later. See section 4.2.2 $ sudo echo 0x00000000000080402 > /sys/class/drm/card0/gvt_disp_ports_mask

### Assigning the port/s to vGPU using UUID

The lspci shows that the system has GFX on pci bus:0, dev:0, func:2.

Please refer to 4.2.2.

# The first occurrence of echoing UUID to create will assign UUID to port number 2 in this example (per “gvt_disp_ports_mask” value) $ sudo echo "d09054ed-30cd-474f-b662-10be479ac004" > /sys/bus/pci/devices/0000\:00\:02.0/mdev_supported_types/i915-GVTg_V5_4/create # The second occurrence of echoing UUID to create will assign UUID to port number 4 in this example (per “gvt_disp_ports_mask” value) $ sudo echo "e49e8cb2-0720-47d0-b035-081ac135d216" > /sys/bus/pci/devices/0000\:00\:02.0/mdev_supported_types/i915-GVTg_V5_4/create # The third occurrence of echoing UUID to create will assign UUID to port number 8 in this example (per “gvt_disp_ports_mask” value) $ sudo echo "6f620412-8c42-42df-a798-a150c97bcfc1" > /sys/bus/pci/devices/0000\:00\:02.0/mdev_supported_types/i915-GVTg_V5_4/create

### De-coupling the ports

The ports assigned to vGPU can be de-assigned by using the following command.

# de-couple ports from the vGPU by writing "1" to "remove" node as below $ sudo echo 1 > /sys/bus/pci/devices/0000\:00\:02.0/d09054ed-30cd-474f-b662-10be479ac004/remove $ sudo echo 1 > /sys/bus/pci/devices/0000\:00\:02.0/e49e8cb2-0720-47d0-b035-081ac135d216/remove $ sudo echo 1 > /sys/bus/pci/devices/0000\:00\:02.0/6f620412-8c42-42df-a798-a150c97bcfc1/remove

# Installing guest OS

Need to clarify (TBD)

The following script will start the guest OS on assigned ports per UUID (GUID) when done correctly. In this example, note the high-lighted UUID (GUID). While the OS is installing, you can connect to the installation process using VNC using port 5900.

- Run the following script to install guest OS

#!/bin/bash -x /usr/bin/qemu-system-x86_64 \ -m 4096 -smp 1 -M q35 \ -enable-kvm \ -name ubuntu-guest \ -hda ~/vm/disk/ubuntu.qcow2 \ -cdrom ~/vm/iso/mini.iso \ -nic user,model=e1000,mac=00:DE:AD:BE:EF:47 \ -vga qxl \ -k en-us \ -vnc :0 \ -machine q35,kernel_irqchip=on \ -cpu host -usb -device usb-tablet \ -device vfio-pci,sysfsdev=/sys/bus/pci/devices/0000:00:02.0/d09054ed-30cd-474f-b662-10be479ac004,display=off,x-igd-opregion=on

- Start VNC client port 5900 to interact with installation process

## Start VM with USB device

Script here is example where you use it as template. Need to make modification to the script per your system specific.

-device usb-host,vendorid=0x045e,productid=0x0800 \

vendorid and productid is your USB device to assign to guest OS such as keyboard.

Type “lsusb” to find the vendorid and productid.

#!/bin/bash -x /usr/bin/qemu-system-x86_64 \ -m 4096 -smp 1 -M q35 \ -enable-kvm \ -name ubuntu-guest \ -hda ./vm/disk/ubuntu.qcow2 \ -nic user,model=e1000,mac=00:DE:AD:BE:EF:47 \ -vga none \ -display egl-headless \ -k en-us \ -vnc :0 \ -machine q35,kernel_irqchip=on \ -cpu host -usb -device usb-tablet \ -device usb-host,vendorid=0x045e,productid=0x0800 \ -device usb-host,vendorid=0x045e,productid=0x07b2 \ -audiodev pa,id=snd0,server=/run/user/1000/pulse/native \ -soundhw hda \ -device vfio-pci,sysfsdev=/sys/bus/pci/devices/0000:00:02.0/404d5798-c07d-11e8-89ef-63dff7a20fd0,display=off,x-igd-opregion=on

## Advanced topic

TDB

### Network in NAT mode

Formatting option for QEMU to add custom network. Make sure you have qemu-ifup-nat file available.

Refer to 4.3.

MAC="00:DE:AD:BE:EF:F1" IFUP=./script/qemu-ifup-nat -net nic,macaddr=$MAC -net tap,script=$IFUP,downscript=no

### Utilization of system

TBD

### Display Switch between Host and VM

By default, the connected display monitor will show content of assigned VM once VGPU
is activated and switch back to host after VGPU is deactivated. To switch display
between host and VM on-the-fly while VM is still running, “gvt_disp_port_owner” can be used. The input value must be in hexadecimal. Refer to section 4.2.3.

Feature for IDV 3.0 only

TBD, add example.

### Device pass-through/assignment to guest OS

#### Passing USB by Bus and Device

The USB device passed to guest OS using Bus and Device is not hot pluggable. The Bus and Device gets assigned whenever USB enumeration happens.

- Find the USB device to pass through/assign to guest OS

#-------------------------------------------------------------- # "lsusb" command will list the USB device has been enumerated #-------------------------------------------------------------- $ lsusb $ lsusb Bus 003 Device 002: ID 046d:c52b Logitech, Inc. Unifying Receiver $

- Formation option for QEMU

Based on the information from “lsusb”, the Unifying Receiver (keyboard and mouse) is enumerated with Bus: 3 and Device 2. The following QEMU option needs to be added as parameter.

-device usb-host,hostbus=3,hostaddr=2

#### Passing USB by vendor and product ID

This method provides limited hot pluggable. When there is multiple device in a system with same vendor and product ID (e.g. same manufacture and model), the behavior is not guaranteed in such case.

- Find the USB vendor and product ID

#-------------------------------------------------------------- # "lsusb" command will list the USB device has been enumerated #-------------------------------------------------------------- $ lsusb $ lsusb Bus 003 Device 002: ID 046d:c52b Logitech, Inc. Unifying Receiver $

- Formation option for QEMU

Based on the information from “lsusb”, the device being pass through/assign to guest OS has vendor and product id as 046d:c52b. The following QEMU option needs to be added as parameter.

-device usb-host,vendorid=0x046d,productid=0xc52b

#### Passing by USB port

This method provides full hot pluggable feature. It passes through the USB port to guest OS rather than USB device.

- Find USB port definition

#-------------------------------------------------------------- # "lsusb -t" command will show USB port information # The port information shows how hardware is wired. #-------------------------------------------------------------- $ lsusb -t $ lsusb -t /: Bus 01.Port 1: Dev 1, class="root_hub", Driver=xhci_hcd/12p, 480M ¦__ Port 3: Dev 3, If 0, class="Hub", Driver=hub/4p, 480M ¦__ Port 4: Dev 18, If 0, class="Human" Interface Device, Driver=usbhid, 1.5M ¦__ Port 3: Dev 17, If 0, class="Human" Interface Device, Driver=usbhid, 1.5M $

- Formation option for QEMU

Based on the information from “lsusb -t”, each USB port has assigned port number. The given USB port is passed and permanently assigned to guest OS for duration of its life. The following QEMU option needs to be added as parameter.

Note: the hostbus and hostport is based on the “lsusb -t” output.

-device usb-host,hostbus=1,hostport=3.3 -device usb-host,hostbus=1,hostport=3.4

#### Passing by PCI device using vfio-pci

QEMU options allow entire PCI device to be pass-through/assigned to guest OS. This process requires binding the PCI devices to IOMMU. Make sure the system BIOS has IOMMU support for VT-x and VT-d. This method need PCI vendor and device identification specified by PCI-SIG organization. Make sure “intel_iommu=on” in the command line option in GRUB and “vfio-pci” is in the module (2.1.1).

- Find PCI device to pass-through

#-------------------------------------------- # "lspci" command to find Bus/Dev/Func #-------------------------------------------- $ lspci $ lspci 01:00.0 USB controller: Renesas Technology Corp. uPD720201 USB 3.0 Host Controller (rev 03) $ Note: The particular PCI device is Bus:01, Dev:00, and Func:0.

- Find the PCI unique device identification for Bus/Dev/Func

#------------------------------------------ # Read the PCI device unique vendor ID #------------------------------------------ $ cat /sys/bus/pci/devices/"0000:01:00.0"/vendor $ cat /sys/bus/pci/devices/"0000:01:00.0"/vendor 0x1912 $ #------------------------------------------ # Read the PCI device unique device ID #------------------------------------------ $ cat /sys/bus/pci/devices/"0000:01:00.0"/device $ cat /sys/bus/pci/devices/"0000:01:00.0"/device 0x0014 $ Note: The PCI device vendor and device ID is for Bus:01, Dev:00, and Func:0.

- Bind the PCI device to IOMMU

#------------------------------------------ # Bind PCI to IOMMU controller # Note: $vendor is 0x1912, and $device is 0x0014 #------------------------------------------ $ echo $vendor $device ¦ sudo tee /sys/bus/pci/drivers/vfio-pci/new_id

- Formation option for QEMU

Based on the information above, the Bus/Dev/Func will be used to assign.

-device vfio-pci,sysfsdev=/sys/bus/pci/devices/0000:00:02.0/01:00.0,x-igd-opregion=on

#### Passthrough MISC devices

Collection of information from other team cross the division

sudo usermod -aG kvm <user> sudo usermod -aG dialout <user>

-audiodev pa,id=snd0,server=/run/user/1000/pulse/native \ -device ich9-intel-hda \ -device hda-duplex,audiodev=snd0 \ -parallel /dev/parportN # replace N with the right number -serial /dev/ttyS0 \ -serial /dev/ttyS1 \ -chardev tty,id=com4,path=/dev/ttyS4 \ -chardev tty,id=com5,path=/dev/ttyS5 \ -chardev tty,id=com6,path=/dev/ttyS6 \ -chardev tty,id=com7,path=/dev/ttyS7 \ -chardev tty,id=com8,path=/dev/ttyS8 \ -chardev tty,id=com9,path=/dev/ttyS9 \ -chardev tty,id=com10,path=/dev/ttyS10 \ -device pci-serial-4x,chardev1=ser2,chardev2=ser2,chardev3=ser3,chardev4=ser4 \ -device usb-serial,chardev=com4 \ -device usb-serial,chardev=com5 \ -device usb-serial,chardev=com6 \ -device usb-serial,chardev=com7 \ -device usb-serial,chardev=com8 \ -device usb-serial,chardev=com9 \ -device usb-serial,chardev=com10 \

### Headless VM setup for Spice

Example script to start the headless guest OS

#! /bin/bash -x NAME="Ubuntu-Guest" #VGPU="f50aab10-7cc8-11e9-a94b-6b9d8245bfc1" MAC="00:DE:AD:BE:EF:20" HDD=./disk/ubuntu-1.qcow2 ISO=./iso/ubuntu.iso BIOS=./fw/bios.bin IFUP=./script/qemu-ifup-bridge DISPLAY="on" VGA="qxl" $EGL="-display egl-headless" /usr/bin/qemu-system-x86_64 \ -m 2048 -smp 2 -M pc \ -name $NAME \ -hda $HDD \ -cdrom $ISO \ -bios $BIOS -enable-kvm \ -net nic,macaddr=$MAC -net tap,script=$IFUP,downscript=no \ -vga $VGA $EGL \ -k en-us \ -serial stdio \ -spice port=5900,disable-ticketing \ -machine kernel_irqchip=on \ -global PIIX4_PM.disable_s3=1 -global PIIX4_PM.disable_s4=1 \ -cpu host -usb -device usb-tablet \

### Automating VM deployment using Portainer

TBD

### Automating VM deployment using LibVirt

TBD

# Remote management using MeshCentral

This section is used to remotely manage the computer might be at remote location such as customer site.

## Basic usage of MeshCentral

- Create account with MeshCentral

Create an account by visiting MeshCentral

- Create a group

Using icons on left column, go to My Account and click “new”

- Add machine you want to gain access

# click "My Device" icon on left column # Look for newly created "Sample-Group" and click "Add Agent" # Look for newly created "Sample-Group" and click "Add Agent" # select the Operating System installed on remote computer # Copy the high-lighted command and paste at remote machine to run

Note: If done correctly, remote machine name should pop-up under the “Sample-Group”. Click it to explore (e.g. Terminal or Files)

## Advance features with MeshCentral

Information was distributed by Joko Sastriawan to gain access to remote machine via MeshCentral using commonly known tools such as “SSH” and “SFTP”.

- Install node.js (https://www.guru99.com/download-install-node-js.html)

- Download mc2router-master from github as zip file (https://github.com/jsastriawan/mc2router)

- Unzip the downloaded file

- Type "npm install nw -g"

- npm i nw -g --nwjs_build_type=sdk

- Type "nw" to start

- Navigate to “File->Configure”

  - Click “proxy”

  - Enter your local proxy information if needed

  - Select the “SSH” tool location

  - Select the “SFTP” tool location

  - Select the “RDP” tool location

  - Select the “Label” in “Custom tool” section (optional)

  - Select the “Path” to custom VNC viewer

  - Add “Arguments”

  - Add “Port”

- Click “Save”

- Click “File->Reset” to log on with your MeshCentral.com account

Note: If done correctly, you should be able to log on to remote machine using SSH. Make sure SSH server is running at remote machine.

# Addendum

Addendum is subject to change/delete. It should be treated as template/example. Customer supporting team should make appropriate change if chose to use.

## Scripts to pull and build kernels

“idv.sh script”

#!/bin/bash -e ################################################################### # version : version of the idv.sh file # cdir : pionts to current working directory where idv.sh file runs # URL : URL to kernel source # BRANCH : tag or branch of the kernel source # PATCHES : idv patch file name with our ".tar.gz" extention ################################################################### version="0.5" cdir=$(pwd) URL="https://git.kernel.org/pub/scm/linux/kernel/git/stable/linux.git" BRANCH="v4.19.65" W_DIR="idv-2.1" PATCHES="idv2.1_er5_patchset" REVISION="2.1" # Display version of idv.sh file echo "idv.sh - version ${version}" # delete all files directorys created by executing idv.sh # Usage: $ ./idv.sh clean function clean() { rm -rf $cdir/$W_DIR rm -rf $cdir/$PATCHES rm -rf $cdir/ubuntu-package rm -rf $cdir/*.deb } [[ "$1" == "clean" ]] && clean && exit 0 #================================================ # Install packages to compile the kernel #================================================ sudo -s <<RUNASSUDO_PACKAGE apt-get autoremove -y &>/dev/null apt-get install -y liblz4-tool kernel-package libelf-dev build-essential git libfdt-dev libpixman-1-dev libssl-dev vim bc socat libsdl1.2-dev libspice-server-dev autoconf libtool xtightvncviewer tightvncserver x11vnc uuid-runtime uuid uml-utilities bridge-utils python-dev liblzma-dev libc6-dev libegl1-mesa-dev libepoxy-dev libdrm-dev libgbm-dev spice-client-gtk libgtk2.0-dev libusb-1.0.0-dev bison flex openssh-server net-tools &>/dev/null RUNASSUDO_PACKAGE #================================================ # Source of Information # https://wiki.ubuntu.com/KernelTeam/GitKernelBuild # IDV 2.1 section 3.2.1 Build Initial Ramdisk #================================================ modules=(kvmgt vfio-iommu-type1 vfio-mdev vfio-pci) for i in "${modules[@]}" do echo $i sudo -s <<RUNASSUDO_MODULES grep -qxF $i /etc/initramfs-tools/modules ¦¦ echo $i >> /etc/initramfs-tools/modules RUNASSUDO_MODULES done cp -a /usr/share/kernel-package $cdir/ubuntu-package ### IDV 2.1 section 3.2.2 Build Kernel Source [[ ! -d "$cdir/$W_DIR" ]] && git clone --depth 1 $URL --branch $BRANCH --single-branch $W_DIR tar xzvf $cdir/$PATCHES.tar.gz cd $W_DIR git reset --hard git apply $cdir/$PATCHES/* &>/dev/null echo ""¦make oldconfig CONCURRENCY_LEVEL=`nproc` fakeroot make-kpkg --initrd --append-to-version=-intelgvt --revision $REVISION --overlay-dir=$cdir/ubuntu-package kernel_image kernel_headers

## Script to pull and build qemu

#!/bin/bash -x cdir=$(pwd) W_DIR="qemu" URL="https://git.qemu.org/git/qemu.git" BRANCH="v4.2.0" [[ "$1" == "clean" ]] && rm -rf $W_DIR && exit 0 function ubu_install_qemu(){ sudo -s <<RUNASSUDO_INSTALL apt purge -y "qemu*" &>/dev/null apt autoremove -y &>/dev/null apt install -y git libfdt-dev libpixman-1-dev libssl-dev vim socat libsdl2-dev libspice-server-dev autoconf libtool xtightvncviewer tightvncserver x11vnc uuid-runtime uuid uml-utilities bridge-utils python-dev liblzma-dev libc6-dev libegl1-mesa-dev libepoxy-dev libdrm-dev libgbm-dev libaio-dev libusb-1.0.0-dev libgtk-3-dev bison &>/dev/null RUNASSUDO_INSTALL if [ ! -d $W_DIR ]; then git clone --depth 1 $URL --branch $BRANCH --single-branch $W_DIR fi cd $W_DIR rm -rf ./roms/seabios git submodule update --init roms/seabios ./configure --prefix=/usr \ --enable-kvm \ --disable-xen \ --enable-libusb \ --enable-debug-info \ --enable-debug \ --enable-sdl \ --enable-vhost-net \ --enable-spice \ --disable-debug-tcg \ --enable-opengl \ --enable-gtk \ --target-list=x86_64-softmmu \ --audio-drv-list=pa if make -j`nproc`; then echo "Make successful" else echo "Make failed" exit 0 fi cd ./roms/seabios make -j `nproc` # make install # cd ../ } ubu_install_qemu

## VFIO PCI unbind and bind script

#!/bin/bash # Usage: pci-vfio.sh <Bus:Dev.Func> if [ "$#" -eq 0 ]; then echo "Usage: pci-vfio.sh Bus:Dev.Func" exit fi for pcie in "$@"; do vendor=$(cat /sys/bus/pci/devices/"0000:$pcie"/vendor) device=$(cat /sys/bus/pci/devices/"0000:$pcie"/device) echo "unbinding -> $pcie ($vendor $device)" if [ -e /sys/bus/pci/devices/"0000:$pcie"/driver ]; then echo "0000:$pcie" ¦ sudo tee /sys/bus/pci/devices/"0000:$pcie"/driver/unbind fi echo "binding -> $pcie ($vendor $device)" echo $vendor $device ¦ sudo tee /sys/bus/pci/drivers/vfio-pci/new_id done

## Gvt.sh (create-vgpu, setup directory)

#!/bin/bash -e #cdir=$(pwd) VM_WORK_DIR=${PWD} cdir=$VM_WORK_DIR #grub_file=/etc/default/grub grub_file=$(pwd)/grub option_GRUB_DEFAULT='"Advanced options for Ubuntu>Ubuntu, with Linux 4.19.65-intelgvt+"' option_CMDLINE='"quiet splash vt.handoff=7 i915.enable_gvt=1 i915.enable_fbc=0 kvm.ignore_msrs=1 intel_iommu=on,igfx_off pcie_aspm=off drm.debug=0"' function clean() { rm -rf $cdir/vm } [[ "$1" == "clean" ]] && clean && exit 0 #-------------------------- # 1) Create working directory #-------------------------- mkdir -p $cdir/vm/disk mkdir -p $cdir/vm/fw mkdir -p $cdir/vm/iso mkdir -p $cdir/vm/script #-------------------------- # 2) Modify GRUB #-------------------------- # setup grub sudo sed -i 's/^GRUB_DEFAULT=.*/GRUB_DEFAULT=\'"$option_GRUB_DEFAULT"'/' $grub_file sudo sed -i 's/^GRUB_CMDLINE_LINUX_DEFAULT=.*/GRUB_CMDLINE_LINUX_DEFAULT=\'"$option_CMDLINE"'/' $grub_file #---------------------------- # Uncomment once testing is done #sudo update-grub2 #---------------------------- #network bridge setup # setup auto start for create vgpu #---------------------------- # prepare the vgpu setup #-------------------------- # 3) Start vgpu-list #-------------------------- #-------------------------- VGPU_LIST=$VM_WORK_DIR/vm/script/vgpu-list touch $VGPU_LIST echo "Adding Mask and GUID, $VGPU_LIST" cat << EOF ¦ sudo tee $VGPU_LIST MASK=0x000000000080402 VGPU0=37414358-bd2b-11e8-8fa3-7bde838751b0 VGPU1=37414358-bd2b-11e8-8fa3-7bde838751b1 VGPU2=37414358-bd2b-11e8-8fa3-7bde838751b2 EOF #-------------------------- #-------------------------- # 4) Start vgpu.service #-------------------------- #-------------------------- SYSTEMD_VGPU=$VM_WORK_DIR/vm/script/vgpu.service touch $SYSTEMD_VGPU echo "Adding VGPU Service $SYSTEMD_VGPU" cat << EOF ¦ sudo tee $SYSTEMD_VGPU [Unit] Description="Start/Stop VGPU service" [Service] WorkingDirectory=/VM_WORK_DIR/ Type=oneshot ExecStart=/bin/bash create-vgpu start ExecStop=/bin/bash create-vgpu stop RemainAfterExit=yes [Install] WantedBy=multi-user.target EOF sed -i "s¦/VM_WORK_DIR¦$VM_WORK_DIR/vm/script¦g" $SYSTEMD_VGPU #######-------- sudo systemd enable $SYSTEMD_VGPU #-------------------------- # 5) Start create-vgpu #-------------------------- # /sys/class/drm/card0 CREATE_VGPU=$VM_WORK_DIR/vm/script/create-vgpu touch $CREATE_VGPU echo "Adding VGPU Service $CREATE_VGPU" cat << EOF ¦ sudo tee $CREATE_VGPU #!/bin/bash -x [[ -f $VGPU_LIST ]] && source $VGPU_LIST echo "mask: \$MASK" echo "vgpu0: \$VGPU0" echo "vgpu1: \$VGPU1" echo "vgpu2: \$VGPU2" echo \$MASK > /sys/devices/pci0000:00/0000:00:02.0/drm/card0/gvt_disp_ports_mask if [ ! -d "/sys/devices/pci0000:00/0000:00:02.0/\$VGPU0" ]; then echo \$VGPU0 > "/sys/devices/pci0000:00/0000:00:02.0/mdev_supported_types/i915-GVTg_V5_4/create" fi if [ ! -d "/sys/devices/pci0000:00/0000:00:02.0/\$VGPU1" ]; then echo \$VGPU1 > "/sys/devices/pci0000:00/0000:00:02.0/mdev_supported_types/i915-GVTg_V5_4/create" fi if [ ! -d "/sys/devices/pci0000:00/0000:00:02.0/\$VGPU2" ]; then echo \$VGPU2 > "/sys/devices/pci0000:00/0000:00:02.0/mdev_supported_types/i915-GVTg_V5_4/create" fi EOF chmod +x $CREATE_VGPU exit 0 sudo systemd enable vgpu.service sudo systemd start vgpu.service #-------------------------- exit 0

## Use case

TBD

## QEMU parameters

TBD